# ISS Telemetry Information

## ISS Data and  Packet Types

ISS  produces two types of data, science and housekeeping.  Table 4-1 outlines the ISS data type by telemetry mode classification. ISS has five packet types which are given in Table 4-2.   Packet types are identical for the NAC and WAC.   Definitions of each packet are as follows:

High Rate Science:  contains the imaging data produced by the camera along with header data.

High Rate Memory Dump:  contains the contents of the ISS memory  and is emitted by  ISS only is response to a READ MEMORY command within the camera(s).

Housekeeping:   contains information on the state of the camera(s), voltages, temperatures, errors, and a brief description of previous images and commands.

HDE Monitor:   contains multiple readings of a single measurement from the housekeeping data electronics (HDE).

Memory Monitor:   contains multiple readings of up to six memory address locations.

The ISS housekeeping packet type telemetry mode is given in Table 4-3.   ISS has a housekeeping data rate of 17.5 bps for any  type of ISS housekeeping packet.

Table 4-1   	Telemetry Mode Classification and ISS Data Type

Classification								ISS Data Types

(1)  Real-time Engineering (RTE)					Housekeeping

(2)  Playback plus Real-time Engineering  (PB&RTE)			Housekeeping

(3)  Probe Checkout (PCHK)						N/A

(4)  Probe Operations (Probe Relay) (PRLY)				Housekeeping

(5)  Science and Engineering Record (S&ER)				Science & Housekeeping

(6)  Real-time Engineering plus Science Playback (RTE&SPB)		Housekeeping

(7)  SAF Instrument Checkout (SAF)					Science & Housekeeping

Table 4-2 	 ISS Packet Types

Type	    	Subaddress		Name				Size (bits)

Science		SCI Data		High Rate Science		7616

Science		SCI Data		High Rate Memory Dump	7616

Housekeeping	SCI HK		Housekeeping			1120

Housekeeping	SCI HK		HDE Monitor			1120

Housekeeping	SCI HK		Memory Monitor		1120

Table 4-3  	Housekeeping Telemetry Mode

Subsystem	Subaddress	   Rate		 Packet	Collection

Size		   Repeat

(bits/sec)	 (bits)		Cycle (sec)

ISS NAC	SCI HK	     17.5		  1120		       64

ISS WAC	SCI HK	     17.5		  1120		       64

Science packet types are produced in only one classification - for Science and Engineering Record.  The names of the principle and default  packets are the High Rate Science packet and the Housekeeping  packet.   In order to obtain the complete set of ISS science telemetry modes and packet types, please note that each SCI Data entry in the subaddress column of Table 4-4 can be either a High Rate Science or a High Rate Memory Dump packet type.

Within the Science and Engineering Record Classification (S&ER), seven of the ten telemetry modes are identified as having ISS actively taking data.  Six of these are during the Optical Remote Sensing (ORS) Operational Mode and the remaining mode is the Optical Navigation Mode.  ISS is in its Operating power state during any ORS mode.

## Detailed Telemetry List

Every packet produced by ISS has the standard 96-bit CCSDS header defined by Cassini Project Document 3-281.    Within this CCSDS header, in bits 11-16, is a Packet Type ID.   Each ISS telemetry packet type and the camera which produced it is identified by this Packet Type ID.  The code is embedded into each packet as part of the Application Process ID (APID).  The whole APID is an 11 bit number which is divided in two parts:  (1) the Subsystem ID and (2) the Packet Type ID.  The ISS Subsystem ID is 01100.  The Packet Type IDs are given in Table 4-5.

### ISS High Rate Science Packet

The ISS high rate science packet contains two types of information, header and image data.  There are three types of headers:   a standard science header, an extended science header, and line headers.  The extended header is located in the first, last and every 50th packet of each image frame produced by the cameras.  [Note:  There can be over 2000 packets per image frame.]  The standard science header is located in all remaining packets of the frame.  The line headers are located at the beginning of each line of image data.  There is only one standard or extended science header per packet.  There may be multiple line headers in a single packet.  The details of these headers are in Table 4-6, Table 4-7, and Table 4-8.  The remainder of each packet is image data.

### ISS High Rate Memory Dump Packet

The ISS high rate memory dump packet is used to read the contents of the entire or a specified area of ISS memory.  The ISS command READ MEMORY will change the science packet to this type.  Details of the packet are in x.

### ISS Measurement Listings for Housekeeping Packets

The details of the ISS housekeeping packets are in Table 4-10 and Table 4-11.  For all items identified as DIGITAL in the listing, a bit definition is provided following the NAC and WAC tables as Table 4-12.   The camera opcodes are in Table 4-13.  The bit definitions apply to both cameras.

### ISS Measurement Listings for HDE Monitor and Memory Monitor Packets

Details of the ISS HDE Packets are in Table 4-14, Table 4-15, and Table 4-16.  Details of the ISS Memory Monitor Packets are inTable 4-17 and Table 4-18.

Table 4-4  	Telemtry Mode Definition for S&ER Classification

ORS Submode		Subsystem	Subaddress	   Rate		Packet                  Collection

Size		Repeat

(bits/sec)	  (bits)                    Cycle (sec)

Stellar Occultation	ISS NAC		SCI Data	 121.9 K	  	  7616		   .125

ISS WAC	SCI Data	See Note	  	  7616		   .125

ISS NAC		SCI Data	 60.9 K	  	  7616		   .125

ISS WAC	SCI Data	 60.9 K	  	  7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Titan			ISS NAC		SCI Data	 60.9 K	  	  7616		   .125

ISS WAC	SCI Data	See Note	  	  7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Saturn Mode 1		ISS NAC		SCI Data	 243.7K	  	  7616		   .125

ISS WAC	SCI Data	See Note	  	  7616		   .125

ISS NAC		SCI Data	 121.9 K	 	  7616		   .125

ISS WAC	SCI Data	 121.9 K	  	  7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Saturn Mode 2		ISS NAC		SCI Data	 60.9 K	  	  7616		   .125

ISS WAC	SCI Data	See Note	  	  7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Icy Satellite Mode 1	ISS NAC		SCI Data	 365.6 K	  	  7616		   .125

ISS WAC	SCI Data	See Note	  	  7616		   .125

ISS NAC		SCI Data	 182.8 K	  	  7616		   .125

ISS WAC	SCI Data	 182.8 K	  	  7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Icy Satellite Mode 2	ISS NAC		SCI Data	 304.6 K	  	  7616		   .125

ISS WAC	SCI Data	See Note	 	   7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Optical Navigation	ISS NAC		SCI Data	365.6		7616		   .125

ISS WAC	SCI Data	365.6		7616		   .125

ISS NAC		SCI HK			See Table 4-3

ISS WAC	SCI HK			See Table 4-3

Note:  The ISS has implemented the "BIU Address Swap" Option which allows the cameras to share the maximum data rate allocated by swapping the NAC and WAC BIU addresses.  This swapping is transparent to the Spacecraft CDS and Ground Data Subsystem.  When ISS BIU Swapping is in effect, the Spacecraft telemetry mode table will carry a non-zero entry for the NAC and a zero entry for the WAC.

Table 4-5  	ISS Detailed Packet Type

| Packet Type ID | APID (Decimal) | Size | Packet Source | Packet Description |
| --- | --- | --- | --- | --- |
| 010001 | 2036.17 | 1120 | SCI HK | ISS NAC Housekeeping |
| 010010 | 2036.18 | 7616 | SCI Data | ISS NAC Science |
| 010111 | 2036.23 | 7616 | SCI Data | ISS NAC Memory Dump |
| 011101 | 2036.29 | 1120 | SCI HK | ISS NAC HDE Monitor |
| 011110 | 2036.30 | 1120 | SCI HK | ISS NAC Memory Monitor |
| 100001 | 2036.33 | 1120 | SCI HK | ISS WAC Housekeeping |
| 100010 | 2036.34 | 7616 | SCI Data | ISS WAC Science |
| 100111 | 2036.39 | 7616 | SCI Data | ISS WAC Memory Dump |
| 101101 | 2036.45 | 1120 | SCI HK | ISS WAC HDE Monitor |
| 101110 | 2036.46 | 1120 | SCI HK | ISS WAC Memory Monitor |

Table 4-6  	ISS Standard Science Header

| Description | Size | Values | Mask |
| --- | --- | --- | --- |
| Camera | 1 | v | 1 |
| Summation (0:1x1, 1:2x2, 2:4x4) | 2 | v | 3 |
| Compression Style | 2 | v | 3 |
| Conversion Type | 2 | v | 3 |
| Line Continue Flag | 1 | v | 1 |
| Science Header Type | 2 | v | 3 |
| Gain | 2 | 0 | 3 |
| Filter 1 Position | 4 | v | 15 |
| Filter 2 Position | 4 | v | 15 |
| Image Line # | 12 | v | 4095 |
| Last Packet Flag (image end flag) | 1 | v | 1 |
| blocks per GroupOfBlock - Lossy | 7 | v | 3 |
| compression rate (Lossy) | 4 | v | XF |
| Malgo (algothim Mode) (lossy) | 1 | v | 1 |
| TB (block Type) (Lossy) | 1 | v | 1 |
| Parity 1 (even parity bit) | 1 | v | 1 |
| Count1(#ofBytes in packet for line) | 15 | v | X7FFF |
| Parity 2 | 1 | v | 1 |
| Count 2 (duplicate of Count1) | 15 | v | X7FFF |
| Extended Pixel, if continueflag=0. Else,this has offset byte# of the line) | 16 | v | XFFFF |
| Overclock Pixel (see Extended) | 16 | v | XFFFF |

Table 4-7  	ISS Extended Science Header

| Description | Size | Values | Mask |
| --- | --- | --- | --- |
| Camera | 1 | v | 1 |
| Summation (0:1x1, 1:2x2, 2:4x4) | 2 | v | 2 |
| Compression Style | 2 | v | 2 |
| Conversion Type | 2 | v | 2 |
| Line Continue Flag | 1 | v | 1 |
| SCIence Header Type | 2 | v | 3 |
| Gain | 2 | v | 3 |
| Filter 1 Position | 4 | v | 15 |
| Filter 2 Position | 4 | v | 15 |
| Image Line # | 12 | v | 4095 |
| Last Packet Flag | 1 | v | 1 |
| Lossy - Block per GroupOfBlock | 7 | v | X7F |
| Lossy - Compression Rate | 4 | v | F |
| Lossy - Malgo (algorithm) | 1 | v | 1 |
| Lossy -TB ( Type Block ) | 1 | v | 1 |
| Cal Lamp | 1 | v | 1 |
| Light Flood. Erase | 1 | v | 1 |
| TCE Heater | 1 | v | 1 |
| TCE Regulate | 1 | v | 1 |
| Optics Heater 1 | 1 | v | 1 |
| Optics Heater 2 | 1 | v | 1 |
| Anti-blooming State | 1 | v | 1 |
| Prepare Index | 4 | v | 15 |
| Readout Index | 4 | v | 15 |
| Table Id | 4 | v | 15 |
| Byte # of Table Entry | 12 | v | 4095 |
| Contents of Entry | 16 | v | XFFFF |
| Image # | 16 | v | XFFFF |
| +50v | 16 | v | XFFFF |
| +30v | 16 | v | XFFFF |
| +28v | 16 | v | XFFFF |
| +12v | 16 | v | XFFFF |
| -12v | 16 | v | XFFFF |
| +5v | 16 | v | XFFFF |
| CCD Temperature | 16 | v | XFFFF |
| Optics 1 Temperature | 16 | v | XFFFF |
| Optics 2 Temperature | 16 | v | XFFFF |
| Optics 3 Temperature | 16 | v | XFFFF |
| Optics 4 Temperature | 16 | v | XFFFF |
| MEA 1 Temperature | 16 | v | XFFFF |
| MEA 2 Temperature | 16 | v | XFFFF |
| MEA 3 Temperature | 16 | v | XFFFF |
| Current (amperes) | 16 | v | XFFFF |
| ID of Last command Triggered | 16 | v | XFFFF |
| # of commands received | 16 | v | XFFFF |
| Current upload | 16 | v | XFFFF |
| FSW state | 8 | v | 255 |
| Exposure Index | 8 | v | 255 |
| Voltage Ref lo | 16 | v | XFFFF |
| Voltage Ref hi | 16 | v | XFFFF |
| spare | 32 | v |  |

Table 4-8  	ISS Line Header

| Description | Size | Values | Mask |
| --- | --- | --- | --- |
| Parity 1 (even parity bit) | 1 | v | 1 |
| Count1(#ofBytes in packet for line) | 15 | v | X7FFF |
| Parity 2 | 1 | v | 1 |
| Count 2 (duplicate of Count1) | 15 | v | X7FFF |
| Extended Pixel, if continueflag=0. Else,this has offset byte# of the line) | 16 | v | XFFFF |
| Overclock Pixel (see Extended) | 16 | v | XFFFF |

Table 4-9  ISS High Rate Memory Dump Packet

| Description | Size | Values | Mask |
| --- | --- | --- | --- |
| Camera | 1 | v | 1 |
| Spare | 7 | 0 | 127 |
| Start Address | 24 | v | XFFFFFF |
| End Address | 24 | v | XFFFFFF |
| HRM Dump Time 1 | 32 | v | X7FFFFFFF |
| HRM Dump Time 2 | 8 | v | 255 |
| Data 1 | 16 | v | XFFFF |
| Data 2 | 16 | v | XFFFF |
| Data 3 | 16 | v | XFFFF |
| Data 462 | 16 | v | XFFFF |
| Data 463 | 16 | v | XFFFF |
| Data 464 | 16 | v | XFFFF |

Table 4-10  	 ISS NAC Housekeeping Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0550 | Present State 1 | 96 | PRES_STATE_1 | DIGITAL | n/a | ISS NAC FSW |
| S-0551 | Present Filter 1 | 104 | PRES_FILT_1 | UNSIGNED | 1-12 | ISS NAC FSW |
| S-0552 | Present Filter 2 | 108 | PRES_FILT_2 | UNSIGNED | 1-12 | ISS NAC FSW |
| S-0553 | Present Shutter | 112 | PRES_SHUTTER | DIGITAL | n/a | ISS NAC FSW |
| S-0554 | Present State 2 | 114 | PRES_STATE_2 | DIGITAL | n/a | ISS NAC FSW |
| S-0555 | Error 1 | 128 | SEU_COUNT | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0556 | Voltage 1 | 136 | HK50V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0557 | Voltage 2 | 148 | HK30V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0558 | Error 2 | 160 | RECOV_ERRORS | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0559 | Voltage 3 | 168 | HK28V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0560 | Voltage 4 | 180 | HK15V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0561 | Error 3 | 192 | FATAL_ERRORS | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0562 | Voltage 5 | 200 | HKMIN15V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0563 | Voltage 6 | 212 | HK5V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0564 | Recoverable Error Type | 224 | LAST_REC_ERR | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0565 | Temperature 1 | 232 | CCD_MON | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0566 | Temperature 2 | 244 | HKOPT1_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0567 | Fatal Error Type | 256 | LAST_FAT_ERR | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0568 | Temperature 3 | 264 | HKOPT2_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0569 | Temperature 4 | 276 | HKOPT3_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0570 | Software | 288 | SOFTWARE | DIGITAL | n/a | ISS NAC FSW |
| S-0571 | Temperature 5 | 296 | HKOPT4_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0572 | Temperature 6 | 308 | HKEFC1_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0573 | Spare 1 | 320 | SPARE_1 | UNSIGNED | n/a |  |
| S-0574 | Temperature 7 | 328 | HKEFC2_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0575 | Temperature 8 | 340 | HKME3_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0576 | Spare 2 | 352 | SPARE_2 | UNSIGNED | n/a |  |
| S-0577 | Current | 356 | ISS_CSENS | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0578 | Spare 3 | 368 | SPARE_3 | UNSIGNED | n/a | TBD |
| S-0579 | RefLo | 372 | VREF_LOW | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0580 | Spare 4 | 384 | SPARE_4 | UNSIGNED | n/a |  |
| S-0581 | Ref Hi | 388 | VREF_HIGH | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0582 | Upload | 400 | UPLOAD_NUM | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0583 | Image Number | 416 | IMAGE_NUM | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0584 | Image 1 | 432 | IMAGE_1 | DIGITAL | n/a | ISS NAC FSW |
| S-0585 | Image 2 | 448 | IMAGE_2 | DIGITAL | n/a | ISS NAC FSW |
| S-0586 | Image 3 | 464 | IMAGE_3 | DIGITAL | n/a | ISS NAC FSW |
| S-0587 | Image 4 | 480 | IMAGE_4 | DIGITAL | n/a | ISS NAC FSW |
| S-0588 | Image 5 | 496 | IMAGE_5 | DIGITAL | n/a | ISS NAC FSW |
| S-0589 | Image 6 | 512 | IMAGE_6 | DIGITAL | n/a | ISS NAC FSW |
| S-0590 | Image 7 | 528 | IMAGE_7 | DIGITAL | n/a | ISS NAC FSW |
| S-0591 | Image 8 | 544 | IMAGE_8 | DIGITAL | n/a | ISS NAC FSW |
| S-0592 | Image 9 | 560 | IMAGE_9 | DIGITAL | n/a | ISS NAC FSW |
| S-0593 | Image 10 | 576 | IMAGE_10 | DIGITAL | n/a | ISS NAC FSW |
| S-0594 | Image 11 | 592 | IMAGE_11 | DIGITAL | n/a | ISS NAC FSW |
| S-0595 | Image 12 | 608 | IMAGE_12 | DIGITAL | n/a | ISS NAC FSW |
| S-0596 | Image 13 | 624 | IMAGE_13 | DIGITAL | n/a | ISS NAC FSW |
| S-0597 | Image 14 | 640 | IMAGE_14 | DIGITAL | n/a | ISS NAC FSW |
| S-0598 | Image 15 | 656 | IMAGE_15 | DIGITAL | n/a | ISS NAC FSW |
| S-0599 | Image 16 | 672 | IMAGE_16 | DIGITAL | n/a | ISS NAC FSW |
| S-0600 | Image 17 | 688 | IMAGE_17 | DIGITAL | n/a | ISS NAC FSW |
| S-0601 | Image 18 | 704 | IMAGE_18 | DIGITAL | n/a | ISS NAC FSW |
| S-0602 | Image 19 | 720 | IMAGE_19 | DIGITAL | n/a | ISS NAC FSW |
| S-0603 | Image 20 | 736 | IMAGE_20 | DIGITAL | n/a | ISS NAC FSW |
| S-0604 | Command Number Received | 752 | COMMAND_REC | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0605 | Command Number Executed | 768 | COMMAND_EXEC | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0606 | Macro | 784 | LAST_MACRO | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0607 | Trigger | 800 | LAST_TRG_CMD | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0609 | Spare 5 | 880 | SPARE_5 | UNSIGNED | n/a |  |
| S-0616 | Command 1 | 816 | LAST_OPCODE1 | DIGITAL | n/a | ISS NAC FSW |
| S-0617 | Command 2 | 824 | LAST_OPCODE2 | DIGITAL | n/a | ISS NAC FSW |
| S-0618 | Command 3 | 832 | LAST_OPCODE3 | DIGITAL | n/a | ISS NAC FSW |
| S-0619 | Command 4 | 840 | LAST_OPCODE4 | DIGITAL | n/a | ISS NAC FSW |
| S-0620 | Command 5 | 848 | LAST_OPCODE5 | DIGITAL | n/a | ISS NAC FSW |
| S-0621 | Command 6 | 856 | LAST_OPCODE6 | DIGITAL | n/a | ISS NAC FSW |
| S-0622 | Command 7 | 864 | LAST_OPCODE7 | DIGITAL | n/a | ISS NAC FSW |
| S-0623 | Command 8 | 872 | LAST_OPCODE8 | DIGITAL | n/a | ISS NAC FSW |

Table 4-11   	ISS WAC Housekeeping Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0700 | Present State 1 | 96 | PRES_STATE_1 | DIGITAL | n/a | ISS NAC FSW |
| S-0701 | Present Filter 1 | 104 | PRES_FILT_1 | UNSIGNED | 1-12 | ISS NAC FSW |
| S-0702 | Present Filter 2 | 108 | PRES_FILT_2 | UNSIGNED | 1-12 | ISS NAC FSW |
| S-0703 | Present Shutter | 112 | PRES_SHUTTER | DIGITAL | n/a | ISS NAC FSW |
| S-0704 | Present State 2 | 114 | PRES_STATE_2 | DIGITAL | n/a | ISS NAC FSW |
| S-0705 | Error 1 | 128 | SEU_COUNT | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0706 | Voltage 1 | 136 | HK50V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0707 | Voltage 2 | 148 | HK30V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0708 | Error 2 | 160 | RECOV_ERRORS | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0709 | Voltage 3 | 168 | HK28V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0710 | Voltage 4 | 180 | HK15V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0711 | Error 3 | 192 | FATAL_ERRORS | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0712 | Voltage 5 | 200 | HKMIN15V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0713 | Voltage 6 | 212 | HK5V_TEL | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0714 | Recoverable Error Type | 224 | LAST_REC_ERR | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0715 | Temperature 1 | 232 | CCD_MON | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0716 | Temperature 2 | 244 | HKOPT1_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0717 | Fatal Error Type | 256 | LAST_FAT_ERR | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0718 | Temperature 3 | 264 | HKOPT2_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0719 | Temperature 4 | 276 | HKOPT3_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0720 | Software | 288 | SOFTWARE | DIGITAL | n/a | ISS NAC FSW |
| S-0721 | Temperature 5 | 296 | HKOPT4_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0722 | Temperature 6 | 308 | HKEFC1_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0723 | Spare 1 | 320 | SPARE_1 | UNSIGNED | n/a |  |
| S-0724 | Temperature 7 | 328 | HKEFC2_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0725 | Temperature 8 | 340 | HKME3_TEMP | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0726 | Spare 2 | 352 | SPARE_2 | UNSIGNED | n/a |  |
| S-0727 | Current | 356 | ISS_CSENS | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0728 | Spare 3 | 368 | SPARE_3 | UNSIGNED | n/a | TBD |
| S-0729 | RefLo | 372 | VREF_LOW | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0730 | Spare 4 | 384 | SPARE_4 | UNSIGNED | n/a |  |
| S-0731 | Ref Hi | 388 | VREF_HIGH | UNSIGNED | 0-4076 | ISS NAC HDE |
| S-0732 | Upload | 400 | UPLOAD_NUM | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0733 | Image Number | 416 | IMAGE_NUM | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0734 | Image 1 | 432 | IMAGE_1 | DIGITAL | n/a | ISS NAC FSW |
| S-0735 | Image 2 | 448 | IMAGE_2 | DIGITAL | n/a | ISS NAC FSW |
| S-0736 | Image 3 | 464 | IMAGE_3 | DIGITAL | n/a | ISS NAC FSW |
| S-0737 | Image 4 | 480 | IMAGE_4 | DIGITAL | n/a | ISS NAC FSW |
| S-0738 | Image 5 | 496 | IMAGE_5 | DIGITAL | n/a | ISS NAC FSW |
| S-0739 | Image 6 | 512 | IMAGE_6 | DIGITAL | n/a | ISS NAC FSW |
| S-0740 | Image 7 | 528 | IMAGE_7 | DIGITAL | n/a | ISS NAC FSW |
| S-0741 | Image 8 | 544 | IMAGE_8 | DIGITAL | n/a | ISS NAC FSW |
| S-0742 | Image 9 | 560 | IMAGE_9 | DIGITAL | n/a | ISS NAC FSW |
| S-0743 | Image 10 | 576 | IMAGE_10 | DIGITAL | n/a | ISS NAC FSW |
| S-0744 | Image 11 | 592 | IMAGE_11 | DIGITAL | n/a | ISS NAC FSW |
| S-0745 | Image 12 | 608 | IMAGE_12 | DIGITAL | n/a | ISS NAC FSW |
| S-0746 | Image 13 | 624 | IMAGE_13 | DIGITAL | n/a | ISS NAC FSW |
| S-0747 | Image 14 | 640 | IMAGE_14 | DIGITAL | n/a | ISS NAC FSW |
| S-0748 | Image 15 | 656 | IMAGE_15 | DIGITAL | n/a | ISS NAC FSW |
| S-0749 | Image 16 | 672 | IMAGE_16 | DIGITAL | n/a | ISS NAC FSW |
| S-0750 | Image 17 | 688 | IMAGE_17 | DIGITAL | n/a | ISS NAC FSW |
| S-0751 | Image 18 | 704 | IMAGE_18 | DIGITAL | n/a | ISS NAC FSW |
| S-0752 | Image 19 | 720 | IMAGE_19 | DIGITAL | n/a | ISS NAC FSW |
| S-0753 | Image 20 | 736 | IMAGE_20 | DIGITAL | n/a | ISS NAC FSW |
| S-0754 | Command Number Received | 752 | COMMAND_REC | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0755 | Command Number Executed | 768 | COMMAND_EXEC | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0756 | Macro | 784 | LAST_MACRO | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0757 | Trigger | 800 | LAST_TRG_CMD | UNSIGNED | 0-32767 | ISS NAC FSW |
| S-0759 | Spare 5 | 880 | SPARE_5 | UNSIGNED | n/a |  |
| S-0766 | Command 1 | 816 | LAST_OPCODE1 | DIGITAL | n/a | ISS NAC FSW |
| S-0767 | Command 2 | 824 | LAST_OPCODE2 | DIGITAL | n/a | ISS NAC FSW |
| S-0768 | Command 3 | 832 | LAST_OPCODE3 | DIGITAL | n/a | ISS NAC FSW |
| S-0769 | Command 4 | 840 | LAST_OPCODE4 | DIGITAL | n/a | ISS NAC FSW |
| S-0770 | Command 5 | 848 | LAST_OPCODE5 | DIGITAL | n/a | ISS NAC FSW |
| S-0771 | Command 6 | 856 | LAST_OPCODE6 | DIGITAL | n/a | ISS NAC FSW |
| S-0772 | Command 7 | 864 | LAST_OPCODE7 | DIGITAL | n/a | ISS NAC FSW |
| S-0773 | Command 8 | 872 | LAST_OPCODE8 | DIGITAL | n/a | ISS NAC FSW |

Table 4-12  	ISS Housekeeping Packet Measurement Bit Definitions,  (1) - (25)

(1)

| S-0550 or S-0700 | Present State 1 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 | Camera | 0: NAC 1: WAC |
| 1-2 | Summation | 00: 1X1 01: 2x2 10: 4x4 11: illegal |
| 3-4 | Compression | 00: no compression 01: lossless 10: lossy 11: illegal |
| 5-6 | Conversion | 00: no conversion 01: 8 lsb 10: 12 to 8 11: illegal |
| 7 | Cal Lamp | 0: off 1: on |

(2)

| S-0553 or S-0703 | Present Shutter |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 | Blade A | 0: reset 1: activated |
| 1 | Blade B | 0: reset 1: activated |

(3)

| S-0554 or S-0704 | Present State 2 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Gain | 00: low 01: high 10: 2x2 sum 11: 4x4 sum |
| 2-5 | Data Rate | TBD |
| 6-7 | MP State | 00: illegal 01: Sleep 10: Idle 11: Active |
| 8 | Light Flood | 0: off 1: on |
| 9 | Anti-Blooming | 0: off 1: on |
| 10 | Optics Heater 1 | 0: off 1: on |
| 11 | Optics Heater 2 | 0: off 1: on |
| 12-13 | TCE State | 00: off 01: on/no regulate 10: off 11: on/regulate |

(4)

| S-0570 or S-0720 | Software |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Software | 00: upload empty 01: uplead full 10: no recent POR 11: POR in last hkp interval |
| 2 - 7 |  | TBD |

Note:  The following  measurement bit definition describe the previous 20 images returned by the camera since POR.

(5)

| S-0584 or S-0734 | Image 1 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Compression | 00: no compression 01: lossless 10: lossy 11: illegal |
| 2 -5 | Filter Pos 1 | [Unsigned] |
| 6 - 9 | Filter Pos 2 | [Unsigned] |
| 10 - 11 | Gain | 00: low 01: high 10: 2x2 sum 11: 4x4 sum |
| 12 - 13 | Conversion | 00: no conversion 01: 8 lsb 10: 12 to 8 11: illegal |

(6)

| S-05845or S-0735 | Image 2 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Compression | 00: no compression 01: lossless 10: lossy 11: illegal |
| 2 -5 | Filter Pos 1 | [Unsigned] |
| 6 - 9 | Filter Pos 2 | [Unsigned] |
| 10 - 11 | Gain | 00: low 01: high 10: 2x2 sum 11: 4x4 sum |
| 12 - 13 | Conversion | 00: no conversion 01: 8 lsb 10: 12 to 8 11: illegal |

(7)

| S-0586 or S-0736 | Image 3 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Compression | 00: no compression 01: lossless 10: lossy 11: illegal |
| 2 -5 | Filter Pos 1 | [Unsigned] |
| 6 - 9 | Filter Pos 2 | [Unsigned] |
| 10 - 11 | Gain | 00: low 01: high 10: 2x2 sum 11: 4x4 sum |
| 12 - 13 | Conversion | 00: no conversion 01: 8 lsb 10: 12 to 8 11: illegal |

•

•

(24)							•

| S-0603 or S-0753 | Image 20 |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 1 | Compression | 00: no compression 01: lossless 10: lossy 11: illegal |
| 2 -5 | Filter Pos 1 | [Unsigned] |
| 6 - 9 | Filter Pos 2 | [Unsigned] |
| 10 - 11 | Gain | 00: low 01: high 10: 2x2 sum 11: 4x4 sum |
| 12 - 13 | Conversion | 00: no conversion 01: 8 lsb 10: 12 to 8 11: illegal |

Note:  The following  measurement bit definition describe the opcodes of the last 8 commands since POR.

(25)

| S-0616 or S-0766 | Command |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 - 7 | Opcode 1 | Various (TBS) |
| 8 - 15 | Opcode 2 | Various (TBS) |
| 16 - 23 | Opcode 3 | Various (TBS) |
| 24 - 31 | Opcode 4 | Various (TBS) |
| 32 - 39 | Opcode 5 | Various (TBS) |
| 40 - 47 | Opcode 6 | Various (TBS) |
| 48- 55 | Opcode 7 | Various (TBS) |
| 56 - 63 | Opcode 8 | Various (TBS) |

Table 4-13  	ISS Command Opcodes

| Opcode | ISS Command |
| --- | --- |
| 0x10 | WAKE |
| 0xe0 | HALT |
| 0x33 | SET_CAMERA |
| 0x43 | CAL_LAMP |
| 0x44 | HDE |
| 0x4a | FILTER |
| 0x4c | FLOOD_LIGHT |
| 0x51 | UPLOAD |
| 0x53 | MONITOR_MEMORY |
| 0x54 | TCE |
| 0x57 | WAIT |
| 0x59 | SHUTTER |
| 0x5a | PORT |
| 0x5b | READ_MEMORY |
| 0x5c | LOAD_MEMORY |
| 0x5e | NOOP |
| 0x60 | LOAD_MACRO |
| 0x64 | LOAD_PMACRO |
| 0x65 | END_MACRO |
| 0x70 | SET_COMP1 |
| 0x76 | SET_COMP2 |
| 0x78 | SET_COMP3 |
| 0x7c | HEATER |
| 0x80 | TRIGGER |
| 0x84 | IMAGE_EVENT |
| 0x85 | ITERATION |
| 0x86 | INSTANT |
| 0xf0 | END_UPLOAD |
| 0xf1 | XSUB |

Table 4-14  	ISS NAC HDE Monitor Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0610 | NAC/WAC | 96 | CAMERA | DIGITAL | n/a | ISS NAC FSW |
| S-0611 | Spare 1 | 97 | SPARE_1 | UNSIGNED | n/a | TBD |
| S-0612 | Code | 104 | HDE_CODE | DIGITAL | ??? | ISS NAC FSW |
| S-0613 | Count | 112 | NUM_VALID | UNSIGNED | 0-62 | ISS NAC FSW |
| S-0614 | Rate | 120 | SAMP_RATE | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0615 | HDE monitor value | 128 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 144 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 160 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 176 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 192 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 208 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 224 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 240 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 256 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 272 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 288 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 304 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 320 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 336 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 352 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 368 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 384 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 400 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 416 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 432 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 448 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 464 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 480 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 496 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 512 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 528 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 544 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 560 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 576 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 592 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 608 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 624 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 640 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 656 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 672 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 688 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 704 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 720 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 736 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 752 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 768 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 784 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 800 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 816 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 832 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 848 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 864 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 880 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 896 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 912 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 928 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 944 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 960 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 976 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 992 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1008 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1024 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1040 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1056 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1072 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1088 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0615 | HDE monitor value | 1104 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |

Table 4-15  	ISS WAC HDE Monitor Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0760 | NAC/WAC | 96 | CAMERA | DIGITAL | n/a | ISS NAC FSW |
| S-0761 | Spare 1 | 97 | SPARE_1 | UNSIGNED | n/a | TBD |
| S-0762 | Code | 104 | HDE_CODE | DIGITAL | 10227 | ISS NAC FSW |
| S-0763 | Count | 112 | NUM_VALID | UNSIGNED | 0-62 | ISS NAC FSW |
| S-0764 | Rate | 120 | SAMP_RATE | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0765 | HDE monitor value | 128 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 144 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 160 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 176 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 192 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 208 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 224 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 240 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 256 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 272 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 288 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 304 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 320 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 336 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 352 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 368 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 384 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 400 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 416 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 432 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 448 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 464 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 480 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 496 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 512 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 528 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 544 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 560 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 576 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 592 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 608 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 624 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 640 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 656 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 672 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 688 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 704 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 720 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 736 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 752 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 768 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 784 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 800 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 816 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 832 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 848 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 864 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 880 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 896 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 912 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 928 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 944 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 960 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 976 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 992 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1008 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1024 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1040 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1056 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1072 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1088 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |
| S-0765 | HDE monitor value | 1104 | HDE Mon Val | DIGITAL | 0-FFFF | ISS NAC FSW |

Table 4-16 	 ISS HDE Packet Measurement Bit Definitions,  (1) - (2)

(1)

| S-0610 or S-0760 | NAC/WAC |  |
| --- | --- | --- |
| Bit | Title | Definition |
| 0 | Camera | 0: NAC 1: WAC |

(2)

| S-0612 or S-0762 | Code |  |
| --- | --- | --- |
| Value | Definition |  |
| 0 | HKTBD3_TEMP |  |
| 1 | HKTBD4_TEMP |  |
| 2 | CCD_MON |  |
| 3 | HKREF_LOW |  |
| 4 | HRREF_HIGH |  |
| 5 | HK50V_TEL |  |
| 6 | HK30V_TEL |  |
| 7 | HK28V_TEL |  |
| 8 | HK15V_TEL |  |
| 9 | HKMIN15V |  |
| 10 | HK5V_TEL |  |
| 11 | ISS_CSENS |  |
| 12 | HKRAD_TEMP |  |
| 13 | HKSHE_TEMP |  |
| 14 | HKOPT1_TEMP |  |
| 15 | HKOPT2_TEMP |  |
| 16 | HKME1_TEMP |  |
| 17 | HKME2_TEMP |  |
| 18 | HKME3_TEMP |  |
| 19 | HKTBD1_TEMP |  |
| 20 | HKTBD2_TEMP |  |

Table 4-17  	ISS NAC Memory Monitor Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0637 | Address Count | 96 | ADDRS_COUNT | UNSIGNED | 1-6 | ISS NAC FSW |
| S-0638 | Count | 104 | NUM_VALID | UNSIGNED | 0-53 | ISS NAC FSW |
| S-0639 | Rate | 112 | SAMP_RATE | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0640 | Spare | 120 | SPARE | UNSIGNED | n/a | TBD |
| S-0641 | Location 1 | 128 | MEM-ADDRS_1 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0642 | Location 2 | 152 | MEM-ADDRS_2 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0643 | Location 3 | 176 | MEM-ADDRS_3 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0644 | Location 4 | 200 | MEM-ADDRS_4 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0645 | Location 5 | 224 | MEM-ADDRS_5 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0646 | Location 6 | 248 | MEM-ADDRS_6 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0520 | 1st & 2nd Mem R/O Values | 272 | Vals 1 + 2 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0521 | 3rd & 4th Mem R/O Values | 304 | Vals 3 + 4 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0522 | 5th & 6th Mem R/O Values | 336 | Vals 5 + 6 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0523 | 7th & 8th Mem R/O Values | 368 | Vals 7 + 8 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0524 | 9th & 10th Mem R/O Values | 400 | Vals 9 + 10 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0525 | 11th & 12th Mem R/O Values | 432 | Vals 11 + 12 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0526 | 13th & 14th Mem R/O Values | 464 | Vals 13 + 14 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0527 | 15th & 16th Mem R/O Values | 496 | Vals 15 + 16 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0528 | 17th & 18th Mem R/O Values | 528 | Vals 17 + 18 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0529 | 19th & 20th Mem R/O Values | 560 | Vals 19 + 20 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0530 | 21st & 22nd Mem R/O Values | 592 | Vals 21 + 22 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0531 | 23rd & 24th Mem R/O Values | 624 | Vals 23 + 24 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0532 | 25th & 26th Mem R/O Values | 656 | Vals 25 + 26 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0533 | 27th & 28th Mem R/O Values | 688 | Vals 27 + 28 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0534 | 29th & 30th Mem R/O Values | 720 | Vals 29 + 30 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0535 | 31st & 32nd Mem R/O Values | 752 | Vals 31 + 32 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0536 | 33rd & 34th Mem R/O Values | 784 | Vals 33 + 34 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0537 | 35th & 36th Mem R/O Values | 816 | Vals 35 + 36 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0538 | 37th & 38th Mem R/O Values | 848 | Vals 37 + 38 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0539 | 39th & 40th Mem R/O Values | 880 | Vals 39 + 40 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0540 | 41st & 42nd Mem R/O Values | 912 | Vals 41 + 42 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0541 | 43rd & 44th Mem R/O Values | 944 | Vals 43 + 44 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0542 | 45th & 46th Mem R/O Values | 976 | Vals 45 + 46 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0543 | 47th & 48th Mem R/O Values | 1008 | Vals 47 + 48 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0544 | 49th & 50th Mem R/O Values | 1040 | Vals 49 + 50 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0545 | 51st & 52nd Mem R/O Values | 1072 | Vals 51 + 52 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0546 | 53rd Mem R/O Value | 1104 | Val 53 | DIGITAL | 0-FFFF | ISS NAC FSW |

Table 4-18  	ISS WAC Memory Monitor Packet

| Channel ID | Description | Position | DMD Title | Type | Range | Source |
| --- | --- | --- | --- | --- | --- | --- |
| S-0787 | Address Count | 96 | ADDRS_COUNT | UNSIGNED | 1-6 | ISS NAC FSW |
| S-0788 | Count | 104 | NUM_VALID | UNSIGNED | 0-53 | ISS NAC FSW |
| S-0789 | Rate | 112 | SAMP_RATE | UNSIGNED | 0-255 | ISS NAC FSW |
| S-0790 | Spare | 120 | SPARE | UNSIGNED | n/a | TBD |
| S-0791 | Location 1 | 128 | MEM-ADDRS_1 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0792 | Location 2 | 152 | MEM-ADDRS_2 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0793 | Location 3 | 176 | MEM-ADDRS_3 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0794 | Location 4 | 200 | MEM-ADDRS_4 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0795 | Location 5 | 224 | MEM-ADDRS_5 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0796 | Location 6 | 248 | MEM-ADDRS_6 | UNSIGNED | 0-524287 | ISS NAC FSW |
| S-0670 | 1st & 2nd Mem R/O Values | 272 | Vals 1 + 2 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0671 | 3rd & 4th Mem R/O Values | 304 | Vals 3 + 4 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0672 | 5th & 6th Mem R/O Values | 336 | Vals 5 + 6 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0673 | 7th & 8th Mem R/O Values | 368 | Vals 7 + 8 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0674 | 9th & 10th Mem R/O Values | 400 | Vals 9 + 10 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0675 | 11th & 12th Mem R/O Values | 432 | Vals 11 + 12 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0676 | 13th & 14th Mem R/O Values | 464 | Vals 13 + 14 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0677 | 15th & 16th Mem R/O Values | 496 | Vals 15 + 16 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0678 | 17th & 18th Mem R/O Values | 528 | Vals 17 + 18 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0679 | 19th & 20th Mem R/O Values | 560 | Vals 19 + 20 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0680 | 21st & 22nd Mem R/O Values | 592 | Vals 21 + 22 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0681 | 23rd & 24th Mem R/O Values | 624 | Vals 23 + 24 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0682 | 25th & 26th Mem R/O Values | 656 | Vals 25 + 26 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0683 | 27th & 28th Mem R/O Values | 688 | Vals 27 + 28 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0684 | 29th & 30th Mem R/O Values | 720 | Vals 29 + 30 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0685 | 31st & 32nd Mem R/O Values | 752 | Vals 31 + 32 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0686 | 33rd & 34th Mem R/O Values | 784 | Vals 33 + 34 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0687 | 35th & 36th Mem R/O Values | 816 | Vals 35 + 36 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0688 | 37th & 38th Mem R/O Values | 848 | Vals 37 + 38 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0689 | 39th & 40th Mem R/O Values | 880 | Vals 39 + 40 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0690 | 41st & 42nd Mem R/O Values | 912 | Vals 41 + 42 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0691 | 43rd & 44th Mem R/O Values | 944 | Vals 43 + 44 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0692 | 45th & 46th Mem R/O Values | 976 | Vals 45 + 46 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0693 | 47th & 48th Mem R/O Values | 1008 | Vals 47 + 48 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0694 | 49th & 50th Mem R/O Values | 1040 | Vals 49 + 50 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0695 | 51st & 52nd Mem R/O Values | 1072 | Vals 51 + 52 | DIGITAL | 0-FFFF FFFF | ISS NAC FSW |
| S-0696 | 53rd Mem R/O Value | 1104 | Val 53 | DIGITAL | 0-FFFF | ISS NAC FSW |

## Bus Interface Unit (BIU) Discrete Data

ISS uses the BIU discrete bits in various ways to report status for ISS PROM events.  The following is an explanation of the tables and when they should be used.

The table titled:  ISS Summary Status of Discrete Data - Maintenance Activity is a summary of the fault codes that were detected during ISS maintenance (exercising the filter wheel and shutter).  A status of 000000 in bits 10-15 indicates that maintenance was performed with no faults.  This table will be used whenever the ISS performs its maintenance activity.  Maintenance occurs primarily during the cruise period.

The table titled:  ISS Summary Status of Discrete Data - Program Loading Activity is a summary of the fault codes that were detected during ISS flight software.  A non-zero status in any of bits 10-15 indicates that the flight software loading process had an error(s).  The summary status will remain in place for 512 seconds before the flight software attempts to execute.  [Note:  For this table, if the loading activity occurs with no faults, the flight software will begin execution immediately and the BIU status is not summarized. After the flight software finishes configuration, it places a code of 000000 in the BIU discrete bits 10-15.]  This table will be used whenever the ISS is turned on and being prepared to take images.  Program loading is currently planned to occur at launch+14 months (checkout) and at approximately SOI-2 years (science cruise).   However, other spacecraft activities may require this activity.

The above referenced tables define only two of the seven PROM activities.  If an unknown anomaly occurs outside of maintenance or program loading, the table titled:  ISS Status of Discrete Data During Execution of PROM Activities needs to be used.  However, the only way this table can be used is if CDS is recording BIU discrete status once per second. [Note:  Currently, the CDS design only records BIU data once per 512 seconds.]  A troubleshooting activity would need to be initiated to acquire this data where ISS is restarted (from the OFF power state) and its discrete data is recorded continuously until all PROM events are completed.  The total time for completion is unknown at this time since it depends on the frequency of other non-ISS spacecraft activities.

4-19	ISS Summary Status of Discrete Data - Maintenance Activity

| Bit | Title | Definition |
| --- | --- | --- |
| 0 | Spare | 0 = “0” 1 = “1” |
| 1 | Instrument Mode | 0 = sleep 1 = active |
| 2 | Operational Mode | 0 = nosafe 1 = safe |
| 3-7 | Spare | 0 = “0” 1 = “1” |
| 8 | Discrete Status 0 : Valid Desdriptor Table Load | 0 = false 1 = true |
| 9 | Discrete Status 1 : FSW is Running | 0 = false 1 = true |
| 10 | Discrete Status 2 | 0 = Filter Wheel 2 returned home |
|  |  | 1 = Filter Wheel 2 couldn't get back to home |
| 11 | Discrete Status 3 | 0 = Filter Wheel 2 left home |
|  |  | 1 = Filter Wheel 2 couldn't get off home |
| 12 | Discrete Status 4 | 0 = Filter Wheel 2 found home |
|  |  | 1 = Filter Wheel 2 couldn't find home |
| 13 | Discrete Status 5 | 0 = Filter Wheel 1 returned home |
|  |  | 1 = Filter Wheel 1 couldn't get back to home |
| 14 | Discrete Status 6 | 0 = Filter Wheel 1 left home |
|  |  | 1 = Filter Wheel 1 couldn't get off home |
| 15 | Discrete Status 7 | 0 = Filter Wheel 1 found home |
|  |  | 1 = Filter Wheel1 couldn't find home |

4-20	ISS Summary Status of Discrete Data - Program Loading Activity

| Bit | Title | Definition |  |
| --- | --- | --- | --- |
| 0 | Spare | 0 = “0” 1 = “1” |  |
| 1 | Instrument Mode | 0 = sleep 1 = active |  |
| 2 | Operational Mode | 0 = nosafe 1 = safe |  |
| 3-7 | Spare | 0 = “0” 1 = “1” |  |
| 8 | Discrete Status 0 : Valid Desdriptor Table Load | 0 = false 1 = true |  |
| 9 | Discrete Status 1 : FSW is Running | 0 = false 1 = true |  |
| 10 | Discrete Status 2 | 0 = ALF packet in sequence |  |
|  |  | 1 = ALF packet out of sequence |  |
| 11 | Discrete Status 3 | 0 = ALF packet message id correct |  |
|  |  | 1 = ALF packet message id bad |  |
| 12 | Discrete Status 4 | 0 = Checksum correct in ALF packet |  |
|  |  | 1 = Checksum error in ALF packet |  |
| 13 | Discrete Status 5 | 0 = Command packet word count correct |  |
|  |  | 1 = Command packet word count wrong |  |
| 14 | Discrete Status 6 | 0 = Total ALF packets correct |  |
|  |  | 1 = Total ALF packets wrong |  |
| 15 | Discrete Status 7 | 0 = Command packet received |  |
|  |  | 1 = Command packet timeout |  |

4-21	ISS Status of Discrete Data During Execution of PROM Activities

| Bit | Title | Definition |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Spare | 0 = “0” 1 = “1” |  |  |  |  |  |
| 1 | Instrument Mode | 0 = sleep 1 = active |  |  |  |  |  |
| 2 | Operational Mode | 0 = nosafe 1 = safe |  |  |  |  |  |
| 3-7 | Spare | 0 = “0” 1 = “1” |  |  |  |  |  |
| 8 | Discrete Status 0 : Valid Desdriptor Table Load | 0 = false 1 = true |  |  |  |  |  |
| 9 | Discrete Status 1 : FSW is Running | 0 = false 1 = true |  |  |  |  |  |
| 10-12 | Discrete Status 2 - 4 : SUROM Non-Fault Status Code | 000 = Initial Condition 010 = Verifying FSW 011 = Exercise Shutter & Filter Wheels 100 = Moving Filter Whl to X Filter Pos 101 = Loading FSW from CDS 110 = Funning FSW 111 = Wait for command |  |  |  |  |  |
|  |  | If Bits 12:10 = |  |  |  |  |  |
|  |  | 000 | 010 | 011 | 101 | 110 |
| 13-15 | Discrete Status 5 - 7 : SUROM Fault Status Code | 111 - Memory Error on Initialization | 001 - Checksum Error 111 - Memory Error on Initialization | 001 - Filter Wheel 1 couldn't find home 010 - Filter Wheel 1 couldn't get off home 011 - Filter Wheel 1 counldn't get back home 100 - Filter Wheel 2 couldn't find home 101 - Filter Wheel 2 couldn't get off home 110 - Filter Wheel 2 counldn't get back home | 001- ALF packet out of sequence 010 - ALF packet message ID bad 011 - Checksum error in ALF packet 100 - Command packet word count wrong 101 - Total ALF packets wrong 110 - Command packet timeout | xxx - flight software loaded but will not run Note: All codes except 000 indicate a fault |

# Camera Operations

## Operating States

### Sleep State

#### How ISS is commanded into SLEEP state

Each camera can be commanded into the sleep state in two ways.  The HALT command can be sent from CDS and the camera will prepare to enter the sleep state.   A controlled shutdown sequence will  be executed if the camera is to transition from an operating state to sleep.  This same response can also be initiated via a discrete sleep bit  in the BIU hardware which acts as a continuous flag to assure sleep will be initiated even if command capabilities are lost.

#### How ISS is commanded out of SLEEP state

To exit the sleep state, the sleep bit  must be reset and the WAKE command must be sent from CDS.

#### Thermal Control

Thermal control of ISS is accomplished via a combination of electrical resistance heaters, thermal insulation, and a passive flat plate radiator.  In sleep  mode, power dissipation within the ISS electronics should provide sufficient heat to maintain a safe operating temperature without the use of the resistance heaters.

#### Memory Contents

The program memory can be loaded from CDS in sleep mode, but the command memory used to execute command sequences is not active and commands cannot be executed in sleep mode.

#### Telemetry

Housekeeping packets are transmitted in sleep.  Science data telemetry is inactive in this mode.

### Idle State

The primary distinction between sleep and idle is the ability in idle mode to load and parse commands.   Commands are executed in the active state, and once executed, the software returns to idle.  (The order of execution is immediate commands first, then triggerable commands in sequence).

#### Memory Contents

The IDLE state is first entered following RAM program load from the CDS.    At a minimum, the Flight Program is present in memory along with any commands which have not yet been executed.   Other memory locations will have been scrubbed during initialization.  The idle state is also entered from the active state, where residual data may have accumulated in the memory during command execution.

#### Thermal Control

Thermal control is provided in the IDLE state in background processing.   Temperature is monitored and the performance heaters are actively controlled to achieve desired operating temperatures within the camera.

#### Telemetry

Housekeeping packets are transmitted in IDLE.   Science data telemetry is inactive in this mode.

### Active State

#### Executing the Sequence

The command sequence is executed in the active state.  IMMEDIATE commands are executed in the order in which they are received.   TRIGGER commands are placed in a temporary command buffer and 	executed each in sequence at a later time as dictated by CDS.    PMACRO  (Permanent MACRO)  commands are placed in a "permanent" command buffer and can be executed (as a sequence) multiple times.

#### Commanding

The ISS accepts and acts upon commands transferred from CDS that contain ISS imaging modes, parameters, and timing.  Commands are transferred as blocks of commands which will be stored in ISS memory.  The minimum size of the transferred blocks is 5 kilowords.  Each block contains imaging commands which are expanded in the ISS into camera electronics commands specifying camera mode, timing and data taking parameters for several frames of ISS data.  The material presented here is introductory in nature.    Details on ISS COMMANDING can be found in the ISS FLIGHT SOFTWARE Architectural Design Document (JPL D-12626).   A command list can also be found later in this document in section 3.2.

#### Frame time

ISS science commanding is on a frame basis which has an associated frame time.  The frame time is split into two major cycles, the prepare cycle and the readout cycle.  The prepare cycle time is used to perform ISS mode changes, filter wheel stepping, heater operation, light flooding, and any other defined functions to prepare for an exposure.  The prepare cycle includes the exposure time.  The ISS also has the ability to take and transmit images with zero exposure time.  The readout cycle is used to collect pixel data, encode or compress the data, and packetize the data.

The ISS prepare cycle is constructed in a structured manner, allowing specific windows of time for performing functions as shown in a simplified timing diagram (Figure 5-1).  Sixteen possible time intervals are available in the prepare cycle.

Figure 5-1  	Simplified Timing Diagram

The total frametime can be determined from a summation of the total time necessary to execute the elements of the two cycles.  The constituent elements are as follows:

PREPARE					READOUT

Shutter Reset					WAC Readout

NAC Filter Wheel Move			NAC Readout

WAC Filter Wheel Move			.625 Sec. Pad

Flood / Erase

Expose

.625 Sec. Pad

##### The Prepare Cycle

The ISS prepare cycle timing is dominated by three items:  the NAC and WAC filter wheel moves and the exposure time.  There are sixteen (16) selections for the prepare cycle which can be made.  These selections, with the associated index used in commanding the camera, are given in Table 5-1.

Table 5-1   ISS Prepare Cycle Index

| Index | NAC Filter Wheel (seconds) | Wac Filter Wheel (seconds) | Exposure (seconds) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 |
| 1 | 2 | 2 | 2 |
| 2 | 3 | 3 | 2 |
| 3 | 5 | 5 | 2 |
| 4 | 5 | 5 | 5 |
| 5 | 5 | 5 | 13 |
| 6 | 5 | 5 | 21 |
| 7 | 5 | 5 | 37 |
| 8 | 5 | 5 | 53 |
| 9 | 5 | 5 | 85 |
| 10 | 5 | 5 | 117 |
| 11 | 5 | 5 | 181 |
| 12 | 5 | 5 | 245 |
| 13 | 5 | 5 | 501 |
| 14 | 5 | 5 | 1013 |
| 15 | 5 | 5 | 1201 |

##### The Readout Cycle

The ISS readout cycle timing is split into two windows:  a NAC readout window and a WAC readout window.  There are sixteen selections for the readout cycle which can be made.  These selections, with the associated index used in commanding the camera, are given in Table 5-2.  This table is valid for a telemetry rate of 48 packets per second (ISS’ highest data rate).  Readout window times in seconds at the other telemetry rates can be calculated by the formula:

Table 5-2  ISS Readout Cycle Index

| Index | NAC Readout (seconds) | WAC Readout (seconds) |
| --- | --- | --- |
| 0 | 50 | 50 |
| 1 | 50 | 25 |
| 2 | 50 | 12 |
| 3 | 50 | 6 |
| 4 | 25 | 50 |
| 5 | 25 | 25 |
| 6 | 25 | 12 |
| 7 | 25 | 6 |
| 8 | 12 | 50 |
| 9 | 12 | 25 |
| 10 | 12 | 12 |
| 11 | 12 | 6 |
| 12 | 6 | 50 |
| 13 | 6 | 25 |
| 14 | 6 | 12 |
| 15 | 6 | 6 |

#### Telemetry

In the active state, both housekeeping and science telemetry can be transmitted to CDS via the ISS BIU's.   Transmittal of science packets via "BIU Swapping" is described in section 2.3.4.2.

##### Data Rates

Five science data rates are available for transmission of ISS science data as given in x-x.  The rate is commanded via CDS and is selected based on a combination of available spacecraft telemetry bandwidth and instrument data priority.

Table 5-3  ISS Data Rates and Associated Packets/RTI

| Data Rate | Packets/RTI |
| --- | --- |
| 365.6 | 6 |
| 304.6 ? | 5 ? |
| 182.8 | 3 |
| 121.9 | 2 |
| 60.9 | 1 |

##### BIU Swapping

In the active state, science packets can be transmitted.  It is a requirement that ALL science packets come from the same BIU address '17'.   This address is associated with the NAC telemetry.  When the WAC sends science packets, theWAC BIU address is temporarily swapped (from ‘18’ to ‘17’) with the NAC BIU address.  The BIU address is only swapped when the camera requiring bus access requests address ‘17’ (See Section ).

##### Packet Types

In the active state, there are three major types of packets which ISS transmits to CDS:  (1) Zero-length 	packets, (2) housekeeping packets, and (3) Science packets.  Zero length packets are used as fill packets for bus transmission continuity.  When CDS detects that a zero length packet has been received, it discards the packet.  For detail on the ISS Housekeeping and Science packets, see section 4.0 of this document.

### Decontamination

TBD

## Camera Features

### Summation

### Gain States

### Anti-Blooming

### Light Flood and Erase

### PC Voltage

### Offset Voltage

### 12 to 8 Bit  Conversion

### Compression

### WAC Calibration Lamp

### Temperature Control

#### Optical Heaters

#### CCD Heater

## Other Camera Values

### Extended and Overclocked Pixels

### Light Flood Duration to Control Residual Bulk Image Effect

### Coefficients for Resistance to Temperature Conversions

## Commanding the Cameras

### Turning On and Waking Up the Cameras

### Loading the Flight Software

### Setting Up the Camera

### The Image Event Command

### Calculating Prepare and Readout Times

### Macros and PMacros

### Loading and Triggering Images

### Commanding Individual Camera Options (Immediate Commands)

### Two Camera Operations

### Shutting Down the Cameras

#### Safing the Filter Wheel and Shutters

#### Entering Sleep Mode/Turning Off

### Other Spacecraft Commands for ISS

## Interpreting Camera Telemetry

### Spacecraft Telemetry

#### Temperature Red and Yellow Alarm Limits

#### BIU Discrete Information During Maintenance

### ISS Packet Telemetry

### ISS Time Tagging

See Claude Hildebrand’s memo

# Idosyncracies

## Filter Wheel Home Position

The ISS filter wheel was designed with two sensors on each wheel.  Since the camera only gets feedback on the filter wheel’s position when it is at “home”, the camera’s software needs to keep track of the wheel’s absolute position.  This is done by counting the number of steps that the stepper motor, driving the filter wheels, is commanded to move.  With this information the software anticipates when the filter wheels are either going to stop or pass through “home” and will check the sensors output to verify that the step count is correct.  When this takes place one of three things can happen:  1) the two sensors turn on when the filter wheel passes through or stops on what the software thinks is “home” and no error message is returned,  2) only one of the sensors turns on when the filter wheel passes through or stops on what the software thinks is “home” and an error message is is returned that says which sensor failed the home test, or  3) neither sensor turns on when the filter wheel passes through or stops on what the software thinks is “home” and an error message is returned that says both sensors failed the home test.

Scenario 2 may happen but should  be taken as an idiosyncracy not a failure of the filter wheel.  The filter wheel is driven by a stepper motor and the tolerance requirement for each sensor was that ‘it indicate “home” with the clear filter in position ± one motor step’.  This means that each sensor could indicate “home” for three steps:  actual “home” and home ± one motor step.  Because of the sensitivity of adjusting the sensors and the dynamic behavior of the filter wheels, the two sensors on the same filter wheel might not agree when a filter wheel was at “home” + 1 step or “home” - 1 step.  This situation was observed during component and subsystem testing on the ground.

## BIU Swapping Final Position and Implications for Follow-on Triggered Images

When the cameras are swapping the BIU address to share the allocated ISS data rate, the addresses are left in the positions of the final swap request.  For example, when performing two camera operations which alternate between the NAC and WAC readouts, the WAC will be the last camera to request a swap.  The address assigned to the WAC, therefore, will be ‘17’.  If the follow-on images are from the NAC, or if a new sequence which requests the NAC be read out first is initiated, the BIU addresses must be swapped before starting this new sequence. The method to accomplish this is to send the command:

[COMMAND]

## Lossy Compression Commanding

# Flight Rules and Constraints