Titan Solar and Stellar Occultations

UVIS acquired about 30 occultations, most of them stellar but several are solar occultations.  Stellar occultations were obtained in both the EUV and FUV channels, whereas solar occultations are only in the EUV channel.  These occultations are of uneven quality determined by the signal/noise, sampling rate, and pointing jitter.  Most of the occultations were obtained while the spacecraft was on thruster control and for those the pointing drift was often large enough to put the star outside of the slit.  Where this occurs can be seen in plots of signal as a function of time, included below.  These plots also give a sense of signal level summed for the entire spectrum.  As can be seen from the plots for UVIS_231ST_EPSORI001_PRIME, three stars in Orion’s sword were observed in different parts of the UVIS slit.

In what follows we give a table listing the occultations, a summary plot showing the ray paths of the whole set of occultations as functions of altitude and latitude and plots showing summed signal versus time for the EUV and (for stellar occultations) FUV.  In the summary plot the line thickness is proportional to the count rate.

Table 1. List of occultation ID, start and end times, and latitude where the ray path crosses the 1000-Km altitude.  Solar occultations have the identifier SU after the orbit number.

________________________________________________________________________

Observation ID				Start Time		End Time		Latitude

UVIS_023ST_BETORI004_PRIME		2006-120T19:42:14	2006-120T20:23:14	 -15.5

UVIS_035ST_ALPERI002_PRIME		2006-346T10:31:31	2006-346T10:57:31	 -35.2

UVIS_037ST_ETAUMA002_ENGR		2007-013T09:37:31	2007-013T10:00:49	  -7.7

UVIS_037ST_ETAUMA002_PRIME		2007-013T10:00:49	2007-013T10:14:31	  -7.7

UVIS_040SU_USUNOCC002_PRIME		2007-069T01:02:00	2007-069T01:38:00	 -76.6

UVIS_046SU_USUNOCC800_PRIME		2007-164T17:39:11	2007-164T18:10:11	 -50.0

UVIS_049ST_SIGSGR002_PRIME		2007-243T05:32:34	2007-243T05:55:34	 -34.6

UVIS_055ST_ALPLYR002_ENGR		2008-005T20:25:20	2008-005T20:46:20	  51.5

UVIS_055ST_ALPLYR002_PRIME		2008-005T20:46:20	2008-005T21:02:20	  51.5

UVIS_059ST_ETACMA002_PRIME		2008-053T19:18:07	2008-053T20:02:07	 -38.8

UVIS_059ST_EPSCMA003_PRIME		2008-054T15:25:07	2008-054T17:55:07	  -5.1

UVIS_059ST_EPSCMA004_PRIME		2008-054T21:20:07	2008-054T23:35:07	 -24.9

UVIS_093ST_ETAUMATI001_PRIME	2008-324T14:58:28	2008-324T15:09:28	   0.8

UVIS_093ST_ETAUMATI002_PRIME	2008-324T15:10:28	2008-324T15:31:28	   0.8

UVIS_093ST_BETCMATI001_PRIME		2008-324T16:20:28	2008-324T16:36:28	  54.2

UVIS_095ST_EPSCMATI001_PRIME		2008-340T15:08:49	2008-340T15:20:45	  19.8

UVIS_107ST_ALPERITI001_PRIME		2009-093T22:27:47	2009-094T00:27:47	  35.6

UVIS_109ST_ALPERITI001_PRIME		2009-109T21:20:45	2009-109T22:45:45	  38.2

UVIS_109SU_USUNOCC001_PRIME		2009-110T00:10:45	2009-110T00:46:45	 -23.2

UVIS_112ST_ETAUMATI001_ENGR		2009-157T20:32:01	2009-157T20:53:10	 -37.4

UVIS_112ST_ETAUMATI002_PRIME	2009-157T20:53:10	2009-157T21:41:01	 -37.4

UVIS_114SU_USUNOCC001_PRIME		2009-189T15:32:03	2009-189T16:37:03	  85.2

UVIS_114ST_ETAUMATI001_ENGR		2009-189T17:30:03	2009-189T17:51:08	  -7.3

UVIS_114ST_ETAUMATI001_PRIME	2009-189T17:51:08	2009-189T18:29:03	  -7.3

UVIS_119SU_USUNOCC001_PRIME		2009-285T06:48:16	2009-285T07:45:25	  -3.6

UVIS_119SU_USUNOCC002_PRIME		2009-285T08:15:25	2009-285T09:01:25	 -66.1

UVIS_153SU_USUNOCC001_PRIME		2011-255T02:23:06	2011-255T03:19:06	  30.9

UVIS_153SU_USUNOCC001_PRIME		2011-255T02:23:06	2011-255T03:19:06	  23.5

UVIS_204ST_ETAUMATI001_PRIME	2014-136T23:12:15	2014-137T02:12:15	  46.4

UVIS_204ST_ETAUMATI002_PRIME	2014-137T07:27:15	2014-137T11:56:15	 -33.8

UVIS_206SU_UTISUNOCC001_PRIME	2014-201T09:55:58	2014-201T11:11:58	 -65.1

UVIS_206SU_UTISUNOCC001_PRIME	2014-201T09:55:58	2014-201T11:11:58	  56.8

UVIS_206ST_ALPERITI001_PRIME		2014-201T11:30:58	2014-201T13:40:58	 -37.4

UVIS_208ST_ETAUMATI001_PRIME	2014-265T00:08:19	2014-265T02:23:19	 -27.7

UVIS_209ST_ALPLYR001_PRIME		2014-294T12:55:00	2014-294T14:15:00  	 -6.1

UVIS_231SU_UTISUNOCC001_PRIME	2016-031T23:48:05	2016-032T01:51:05	 -63.8

UVIS_231SU_UTISUNOCC001_PRIME	2016-031T23:48:05	2016-032T01:51:05	  49.9

UVIS_231ST_EPSORI001_PRIME		2016-032T01:51:05	2016-032T03:05:05	  33.7

UVIS_231ST_EPSORI001_PRIME		2016-032T01:51:05	2016-032T03:05:05	   7.1

UVIS_231ST_EPSORI001_PRIME		2016-032T01:51:05	2016-032T03:05:05	  33.7

UVIS_231ST_EPSORI001_PRIME		2016-032T01:51:05	2016-032T03:05:05	   7.1

UVIS_231ST_EPSORI001_PRIME		2016-032T01:51:05	2016-032T03:05:05	  33.7

UVIS_234SU_UTISUNOCC001_PRIME	2016-095T18:57:42	2016-095T19:12:42	 -58.0

UVIS_234SU_UTISUNOCC005_PRIME	2016-095T19:13:42	2016-095T19:51:42	 -58.0

UVIS_234SU_UTISUNOCC006_PRIME	2016-095T20:13:42	2016-095T20:47:42	  59.1

________________________________________________________________________

Summary Plot

Plots of signal level (counts per integration) as a function of time commence here.